Angle-of-Arrival Estimation with a

Mechanically Rotating Single Wi-Fi Antenna

Problem Formulation and MUSIC / ESPRIT / SAGE Solutions

# 1. Purpose and Scope

This note formulates, from first principles, how a single Wi-Fi antenna that is mechanically rotated about a fixed base can be used to estimate the angle of arrival (AoA) of one or more incoming signals, and then derives three classical estimators — MUSIC, ESPRIT, and SAGE — for two rotation geometries:

- Scenario A — 1 rotational degree of freedom (DoF): the antenna is pinned at its base and tilts in a single vertical plane, stepping 5° from 0° to 180° (horizontal on one side, up through vertical, to horizontal on the other side).

- Scenario B — 2 rotational DoF (pan-tilt gimbal): the antenna can be tilted to any elevation and, at that tilt, panned in azimuth, so its boresight can point anywhere on the upper hemisphere. The base is the rotation origin.

The physical idea in both cases is the same: mechanical rotation turns one antenna, sampled sequentially over time, into a synthetic array. The document first builds the shared signal model and states the core measurement challenges (the "problem construction"), then treats each scenario in turn. Assumptions: a common consumer Wi-Fi element with a dipole-like (omni-in-azimuth, toroidal / donut-shaped) radiation pattern; an OFDM Wi-Fi waveform (2.4 / 5 / 6 GHz) that exposes per-subcarrier channel state information (CSI); and quasi-static geometry during one sweep.

# 2. Shared Signal and Geometry Model

## 2.1 Antenna pattern and phase center

Let the antenna body frame have its axis along ẑ_body. A typical half-wave dipole has the (magnitude) field pattern

F(χ) = | cos( (π/2)·cos χ ) / sin χ |

where χ is the angle between the antenna axis and the incoming direction. F(χ) is maximum broadside (χ = 90°) and has deep nulls along the axis (χ = 0°, 180°). Two features of the element matter for AoA: (i) its directional gain g(χ) = F(χ)·e^{jΦ(χ)} modulates the received amplitude as the antenna rotates, and (ii) its phase center sits a distance ρ from the base along the antenna axis, so rotation physically displaces the phase center and creates path-length (phase) differences. The amplitude cue dominates when ρ ≪ λ; the phase (synthetic-aperture) cue grows as ρ approaches λ. For a quarter/half-wave element at 2.4 GHz (λ ≈ 12.5 cm) the arc radius ρ is a fraction of λ, so a good estimator should exploit amplitude and phase jointly.

## 2.2 Received-signal model

Index the rotation positions by k = 1…K (orientation Ω_k), OFDM subcarriers by n = 1…L (wavelength λ_n), and repeated sweeps / symbols by t = 1…M. After removing the known pilot/preamble symbol s[n,t] (WiFi LTF), the measured complex CSI sample for a single far-field path from direction u(θ,φ) is

x_k[n,t] = g( χ_k(θ,φ) ) · exp( j·(2π/λ_n)·p_kᵀ·u(θ,φ) ) · γ + w_k[n,t]

where p_k is the phase-center position at orientation Ω_k, γ the complex path gain, and w white noise. Stacking the K positions into one synthetic snapshot x = [x_1,…,x_K]ᵀ and allowing D superimposed paths (line-of-sight plus reflections) gives the array model

x = Σ_{d=1..D} γ_d · a(θ_d, φ_d) + w  =  A·γ + w

with steering vector a(θ,φ) = [a_1,…,a_K]ᵀ, a_k(θ,φ) = g(χ_k)·exp( j·(2π/λ)·p_kᵀ·u ), manifold matrix A = [a(θ_1,φ_1),…,a(θ_D,φ_D)], and gain vector γ. This A·γ + w form is exactly the classical array-processing model — MUSIC, ESPRIT, and SAGE all operate on it — except that here the "array" is synthesised by one moving element and the manifold a(·) carries the real antenna pattern g, not just an ideal phase term.

## 2.3 Problem construction — the four challenges unique to a rotating single antenna

- Sequential (non-simultaneous) sampling. The K synthetic elements are visited one at a time, not captured in one snapshot. Treating them as a single vector x requires the channel and source to stay coherent across the whole sweep (quasi-static assumption). Sweep-time budget therefore trades against Doppler/scene motion.

- Phase-coherence / reference. Combining positions by phase demands a common phase reference across the sweep; otherwise transmitter phase, CFO and SFO drift corrupt p_kᵀu. Practical fixes: keep a second fixed reference antenna receiving simultaneously and use the ratio x_k / x_ref; or exploit the repeated WiFi LTF pilots to track and remove residual phase. Without a reference, fall back to amplitude-only (|x_k|) processing — lower resolution but robust.

- Coherent multipath. Indoor reflections are fully correlated with the LoS path, which rank-deficits the covariance and breaks subspace methods. Remedies: spatial smoothing / forward-backward averaging (in phase-mode / beamspace for the circular manifold), or use SAGE, which is a parametric maximum-likelihood method that handles coherent paths natively.

- Pattern and phase-center calibration. The estimators need an accurate measured g(χ) and ρ. Manufacturing spread, the radome, and near-field coupling to the AP chassis distort the pattern, so a per-unit calibration table (or an on-line self-calibration term) is required.

# 3. Scenario A — Single-Axis (1-DoF) Planar Rotation

## 3.1 Geometry and rotation model

Place the base at the origin O and let the antenna tilt in the x–z (vertical) plane about the ŷ axis. With step Δ = 5° and k = 0…36 (K = 37 positions):

β_k = k·Δ ,  Δ = 5° ,  k = 0…36   (β from 0° to 180°)

antenna axis:  d̂(β) = ( cos β , 0 , sin β )

phase center:  p_k = ρ · d̂(β_k)

A source lying in the scan plane has direction u(ψ) = (cos ψ, 0, sin ψ), where ψ ∈ [0°,180°] is the in-plane arrival angle to be estimated. The angle between the antenna axis and the source is simply χ_k = β_k − ψ, so both the pattern and the phase term depend only on the difference (β_k − ψ):

a_k(ψ) = F(β_k − ψ) · exp( j·(2πρ/λ)·cos(β_k − ψ) )

This is a circular-array manifold. Its key structural property: the response as a function of orientation is a fixed "template" F(·)·exp(j(2πρ/λ)cos(·)) that is simply shifted by the unknown AoA ψ. For a single source, AoA estimation is therefore matched filtering — correlate the measured curve against the template and read off the shift. MUSIC/ESPRIT/SAGE generalise this to multiple superimposed paths.

Note — dimensionality.  One rotation axis resolves only the arrival angle projected into the scan plane; an out-of-plane source appears at its projected angle. Full 2-D AoA (azimuth + elevation) requires the 2-axis system of Section 4.

Note — sampling.  To avoid spatial aliasing on the synthetic arc, the inter-step arc length must satisfy ρ·Δ(rad) ≤ λ/2, i.e. ρ ≤ λ/(2·0.087) ≈ 5.7λ for Δ = 5°. For a physical element ρ ≪ λ this is amply satisfied; 5° in fact oversamples, giving margin to average or to coarsen the sweep.

## 3.2 Building snapshots and the covariance

Subspace methods need a covariance estimate of full signal rank. Because one sweep yields a single synthetic vector x, obtain multiple snapshots by (i) repeating the sweep M times, (ii) using the L OFDM subcarriers as frequency snapshots (recomputing a(ψ) per λ_n, or applying wideband focusing matrices), and (iii) forward–backward / phase-mode spatial smoothing to decorrelate coherent multipath:

R̂ = (1/(M·L)) · Σ_{m,n}  x[m,n] · x[m,n]ᴴ   ∈ ℂ^{K×K}

## 3.3 MUSIC

Eigendecompose the covariance and split into signal (D largest eigenvalues) and noise subspaces:

R̂ = U_s Λ_s U_sᴴ + U_n Λ_n U_nᴴ

The AoA estimates are the peaks of the 1-D MUSIC pseudo-spectrum over ψ ∈ [0°,180°] (a 37-point grid or finer):

P_MUSIC(ψ) = 1 / ( a(ψ)ᴴ · U_n · U_nᴴ · a(ψ) )   →   ψ̂ = argmax_ψ P_MUSIC(ψ)

Because the manifold a(ψ) embeds the true antenna pattern, MUSIC here automatically fuses the amplitude (pattern) and phase (aperture) cues. The number of resolvable paths is limited by K and by how well multipath has been decorrelated.

## 3.4 ESPRIT

ESPRIT needs a shift-invariant (Vandermonde) manifold, which a circular arc does not have directly. The standard route is a phase-mode / beamspace transform (the Davies transformation used in UCA-ESPRIT): a DFT across the uniform 5° samples maps the circular manifold, whose modes are weighted by Bessel functions J_m(2πρ/λ), onto a virtual uniform linear array with entries e^{jmψ}:

Tᴴ · a(ψ) ≈ diag{ Jₘ(2πρ/λ) } · [ e^{−jMψ}, …, e^{+jMψ} ]ᵀ

On the transformed manifold the usual rotational-invariance step applies. With selection matrices J₁, J₂ picking overlapping mode subsets and the transformed signal subspace E_s:

J₁ · E_s · Ψ = J₂ · E_s   →   Ψ = (J₁E_s)⁺ (J₂E_s)

eig(Ψ) = e^{ jψ_d }   →   ψ̂_d = arg( eig )

ESPRIT gives the AoAs in closed form (no spectral search), at the cost of the beamspace approximation and sensitivity to the accuracy of ρ and of the uniform-angular-sampling assumption.

## 3.5 SAGE (recommended for this problem)

SAGE (Space-Alternating Generalized Expectation-maximization) is a maximum-likelihood estimator that fits the D paths one at a time and is the most natural fit for a rotating single antenna: it works with few (even one) snapshots, tolerates coherent multipath, uses the measured pattern g directly (no idealised manifold), and jointly estimates amplitude and angle. Model the K measurements as

x = Σ_{d=1..D} γ_d · a(ψ_d) + w ,   w ~ CN(0, σ²I)

SAGE decomposes the observation into per-path "admissible hidden data" and alternates:

- Initialise (ψ_d, γ_d) for d = 1…D by successive interference cancellation using the beamformer / MUSIC peak.

- E-step: form the complete data for path d by adding back its share of the residual — ŷ_d = x − Σ_{d'≠d} γ_{d'} a(ψ_{d'}).

- M-step (angle): ψ̂_d = argmax_ψ | a(ψ)ᴴ ŷ_d |² / ‖a(ψ)‖² — a cheap 1-D search over the scan range.

- M-step (gain): γ̂_d = a(ψ̂_d)ᴴ ŷ_d / ‖a(ψ̂_d)‖².

- Cycle over d = 1…D until the log-likelihood converges; optionally add delay τ_d (from OFDM subcarrier phase slope) and Doppler ν_d for moving clients, giving joint angle-delay-Doppler estimates.

Because SAGE evaluates the exact pattern-plus-phase manifold, it degrades gracefully when the aperture is tiny (falls back to pattern/amplitude information) and when only magnitudes are trustworthy (magnitude-only likelihood).

# 4. Scenario B — Two-Axis (Pan–Tilt) Rotation

## 4.1 Completing the spatial rotation model

Give the base a 2-axis gimbal: a tilt (elevation) α about ŷ and a pan (azimuth) γ about ẑ. Using the intrinsic rotation R(γ,α) = R_z(γ)·R_y(α) applied to the body axis ẑ_body, the boresight and phase-center trace a sphere of radius ρ:

b̂(γ,α) = R_z(γ) R_y(α) ẑ = ( sin α cos γ , sin α sin γ , cos α )

p(γ,α) = ρ · b̂(γ,α)

Parameterise the unknown source direction by elevation θ and azimuth φ:

u(θ,φ) = ( sin θ cos φ , sin θ sin φ , cos θ )

The angle between boresight and source, and hence the full 2-D steering manifold over the two control angles, is

χ(γ,α; θ,φ) = arccos( b̂(γ,α)ᵀ u(θ,φ) )

a(θ,φ; γ,α) = g( χ ) · exp( j·(2π/λ)·ρ· b̂(γ,α)ᵀ u(θ,φ) )

As in Scenario A the response depends on the mismatch between where the antenna points and where the source is (through χ and through b̂ᵀu = cos χ), so for a single source it is again a 2-D template-matching / peak-finding problem; for multipath it is the full 2-D array model. Practical modelling points: use intrinsic Euler angles and restrict α ∈ [0°,90°], γ ∈ [0°,360°) to cover the upper hemisphere while avoiding gimbal-lock degeneracy near α = 0° (where γ is unobservable); and pre-compute a calibrated a(θ,φ;γ,α) lookup because the two-axis pattern is not separable in general.

## 4.2 Sampling the sphere

A comprehensive raster over (γ,α) should keep neighbouring pointings within roughly half the element beamwidth (and satisfy the ρ·Δ ≤ λ/2 arc-sampling rule per axis) so that no lobe of the pattern is skipped. An equal-area or spiral spherical grid gives more uniform angular coverage than a naïve lat/long raster, which oversamples near the pole (α ≈ 0°).

## 4.3 Two operating modes: fast few-measurement search vs. comprehensive scan

The 2-axis freedom creates a genuine choice between finding one dominant source as fast as possible and mapping the whole angular field.

### (a) Fast AoA from one / a few measurements — active pointing

For a single strong client (e.g., a latency-sensitive LoS device) a full raster is wasteful. Because the received power P(γ,α) is a smooth unimodal-ish function of pointing near the source, the antenna can climb to the source in a handful of steps:

- Sequential lobing / gradient ascent: from the current pointing, probe a few offsets (±δ in γ and in α), estimate the local gradient of P by finite differences, and step toward increasing power; repeat with shrinking δ (coarse-to-fine). This is the single-antenna analogue of monopulse (true monopulse needs simultaneous sum/difference beams, which one element cannot form).

- Null-based refinement: a dipole's pattern null is far sharper than its broad peak, so once near the source, rotate to place a pattern null on it and use the null depth for a high-precision angle — sharper than peak-seeking.

- Bayesian active sensing: maintain a posterior p(θ,φ | measurements) and choose the next orientation that maximises expected information gain (minimises posterior entropy) — an information-optimal few-shot search; Thompson sampling is a lightweight approximation.

With a directional element these methods localise a dominant source in on the order of 5–15 measurements instead of a full ~hundreds-point raster. The trade-off: they assume a single dominant path and can be misled by strong reflections, so they suit tracking a known client, not discovering the environment.

### (b) Comprehensive scan — full spatial sensing

Rastering the whole (γ,α) grid and applying a subspace / ML estimator yields the complete angular power (and phase) map: all significant paths, their AoAs and relative strengths, and therefore the dominant reflectors and the multipath geometry of the room. This is what feeds multi-client optimisation and environment characterisation. It costs more sweep time and assumes the scene is static for the duration.

Guidance.  Use fast active pointing to acquire/track a single high-priority client or to re-lock after a small move; use a comprehensive scan (periodically or on trigger) to build the multi-path spatial map used for multi-client orientation optimisation and for detecting new reflectors.

## 4.4 MUSIC, ESPRIT, and SAGE in 2-D

MUSIC (2-D): build R̂ over the sampled orientations (plus subcarrier/sweep snapshots), and search the pseudo-spectrum over the (θ,φ) grid:

P_MUSIC(θ,φ) = 1 / ( a(θ,φ)ᴴ U_n U_nᴴ a(θ,φ) )

ESPRIT (2-D): decompose the spherical manifold into spherical-harmonic / phase modes (a 2-D beamspace transform), which yields two shift-invariant structures — one per angular coordinate — so that Unitary / EB-ESPRIT returns azimuth and elevation in closed form with automatic pairing. Fast, but relies on the harmonic approximation and accurate calibration.

SAGE (2-D, recommended): the same per-path coordinate-ascent as §3.5, now maximising over the pair (θ_d, φ_d), and optionally over delay τ_d (OFDM) and Doppler ν_d. SAGE remains the workhorse for the rotating-antenna problem: it copes with the irregular spherical sampling, coherent multipath, few snapshots, and the real (non-separable, calibrated) 2-axis pattern, and it produces the parametric path list that both operating modes above can consume.

# 5. Practical Wi-Fi Implementation Notes

- CSI as free snapshots. Each WiFi packet's LTF gives CSI on L subcarriers; across a 20/40/80/160 MHz channel these act as frequency snapshots and also let SAGE estimate path delay from the phase-vs-subcarrier slope.

- Reference antenna. A second, fixed antenna receiving the same packet provides the phase reference that makes cross-position phase meaningful; process x_k / x_ref.

- Sweep timing. Budget the mechanical settle-plus-measure time per step against channel coherence; for moving people include Doppler in SAGE or shorten/interleave the sweep.

- Calibration. Store a measured g(χ) (and ρ, and the 2-axis pattern for Scenario B) per antenna; re-estimate a residual complex calibration term on-line from a known-direction anchor if available.

- Fallback. If phase coherence cannot be guaranteed, run amplitude-only pattern-matching (or magnitude-domain SAGE): coarser, but requires no reference and no phase tracking.

# 6. Estimator Comparison

| Method | Principle | Snapshots | Coherent MP | Output | Best use here |
| --- | --- | --- | --- | --- | --- |
| MUSIC | Noise-subspace spectral search | Needs many (R̂ rank) | Needs smoothing/FBA | Pseudo-spectrum peaks | Visualising the angular map; multi-path survey |
| ESPRIT | Rotational invariance (beamspace) | Moderate | Needs smoothing | Closed-form angles | Fast closed-form angle read-out after beamspace transform |
| SAGE | Per-path maximum likelihood (EM) | Few / even one | Handled natively | Parametric path list (angle, gain, delay, Doppler) | Primary estimator: irregular sampling, real pattern, coherent MP |

# 7. Notation

| Symbol | Meaning |
| --- | --- |
| k, K | rotation-position index / total positions (Scenario A: K = 37 at 5° steps) |
| β_k | single-axis tilt angle (Scenario A) |
| γ, α | pan (azimuth) and tilt (elevation) control angles (Scenario B) |
| θ, φ / ψ | source elevation & azimuth (2-D) / in-plane AoA (1-D) |
| u(θ,φ) | unit vector toward the source |
| b̂, d̂ | antenna boresight / axis direction |
| ρ | phase-center offset from base (arc radius) |
| λ, λ_n | wavelength / subcarrier n wavelength |
| g(χ), F(χ) | complex element gain / dipole magnitude pattern; χ = angle to source |
| a(·) | synthetic-array steering vector (pattern × phase) |
| γ_d, D | complex gain of path d / number of paths |
| R̂, U_s, U_n | sample covariance; signal / noise subspace |
| M, L | repeated sweeps / OFDM subcarriers (snapshot sources) |

RSSI